Classifying common hand gestures using gyroscopic sensor data from a mobile device

Ashley Hunt 
Computer Science
University of Nottingham

Amit Kumar
Data Science
University of Nottingham

Shreeya Kumbhoje
Data Science
University of Nottingham

In this paper we attempt to utilise machine learning (ML) models and recurrent neural networks (RNN) to classify four common hand gestures (1. Drawing a circle, 2. Waving, 3. Gesturing come here, 4. Gesturing go away). These gestures were recorded using the gyroscopic sensors inside a mobile phone. Our aim here is to train the computer using the labelled data so that it can classify between different hand gestures the user performs. The raw gyroscopic data goes through a set of preprocessing techniques such as the application of a low-pass filter, Fourier transformation and then a feature extraction and selection process to optimise the data provided for the training of the model. We have deployed ML models including K-means Classifier, Support Vector Machines (SVM), Random Forest Classifier, XGBoost, K-Nearest Neighbour (KNN) and the Recurrent Neural Network Models Long Short-Term Memory layer (LSTM) and Gated Recurrent Unit (GRU) looking for an optimum fit model for multi class classification between gestures.

The outcome shows a good  __% accuracy for ___ ML model in comparison with other ML models implemented, and the accuracy of __% LSTM(RNN model) and ___% for GRU(RNN model).

# Introduction

Gesture recognition pertains to recognizing meaningful expressions of motion by a human, involving the hands, arms, face, head, and/or body[8]. With a variety of application across different fields including monitoring activities and vital signs, enhancing gaming experience, and supporting applications in robotics and wearable technology. Our  intent is to use smartphones which is a practically used device to establish  communication possibilities between humans and machines differing from the traditional data collection methods. we further explore the usage of this technology to create accessibility features for individuals with disabilities which could be a potential project within the scope of man machine interaction.

In recent years, gesture recognition system has become very popular in the field of research, especially facial and hand gesture recognition system[3] gesture classification for hand movement can be classified into static and dynamic depending on the data acquisition method.  In the past a significant amount of research has been carried out which investigates machine algorithms like Support Vector Machine[1][3] , Random Forest[1] , Hidden Markov Model which analyses data to discover patterns. A  few Deep Learning models like CNN[1][6] and some hybrid models like CNN-LSTM[1] mostly focusing on  image processing, which has now made it a common subject in the domain. However, a limited amount of research focuses on using signal data collected as a .dataset(.csv file)  using a normally accessible device among humans today.

# Background

Quote dump:

"The highest performance was achieved using a convolutional neural network with long-short term memory (CNN-LSTM)"

"The pre-processing step consisted of a fourth-order low-pass Butterworth filtering with a cut-off frequency of 5 Hz. Such frequency was chosen due to the frequency content of the acquired signals, which was below 5–6 Hz, as verified by Fourier power spectrum analysis. The pre-processing also included raw signal standardization: each signal acquired with the MMR sensors was centred to have zero mean and standard deviation equal to one."

"Concerning ML approaches, inspired by human activity recognition work in the literature, the following classifiers were evaluated: Support Vector Machine (SVM) [19,20], Random Forest (RF) and K-Nearest Neighbour (KNN)."

"The complete dataset, composed of 22 subjects, was divided into training/validation and test sets using leave-one-subject-out cross validation. As concerns ML methods, the training/validation dataset was split into 5 folds, and grid search-based 5-fold cross validation was used for tuning the hyperparameters. As concerns DL methods, validation was performed on 20% of the training/validation set."

"In multi-gesture classification, CNN and CNN-LSTM performed better than ML methods, with CNN-LSTM resulting in the highest balanced accuracy"

"In the binary classification, the highest f1-score and precision values were obtained with CNN" (Moccia, Sara et al.)

"Support vector machine (SVM) and hidden Markov model (HMM) are two typical machine learning methods in pattern recognition."

"SVM usually needs careful feature extraction and selection, as well as other traditional ML algorithms like naïve Bayes (NB), K-nearest neighbors (KNN), and decision tree (DT)"

"For time sequential signals, recurrent neural networks (RNN) allow information to persist. Thus, we can make full use of the contextual information of the sequence. However, RNN might suffer from a vanishing gradient problem with long data sequences. To solve this problem, Hochreiter et al. designed a special RNN, the long short-term memory (LSTM) network that can learn long dependencies" (Zhao, Shenglin et al.)

# Methodology

To achieve our aims of producing a classification model for four gestures (come, go, wave and circle), the raw gyroscopic data is heavily pre-processed and undergoes several key transformations to contribute to the classification process. Only the most useful features are taken through to the model training phase.

## Data Collection

Data was collected using the Phyphox mobile application. Each member of the team attempted to perform the same four gestures for eight iterations per recording, and five recordings each per gesture – 40 iterations per person per gesture for a total of 480 complete gestures.

These gestures were initially recorded continuously, without stopping, as per the coursework specification but after trial and error we discovered that segmenting the data was a lot more difficult when there were no clear spaces between the gestures. Instead, we opted to leave a distinct one second gap between performing each gesture in each recording. Adding these pauses allowed us to segment the recording more easily into its separate gesture patterns.

Figure 1: Screenshot of the Phyphox recording interface.

We imported two separate data sets, one for testing and training and the other for validation. Both data sets would undergo the same pre-processing, but we would apply a train_test split to the first and the second would be kept away from the model until we needed to perform validation on the model performance.

## Data Visualization

Once the data was recorded and the associated csv files were imported into pandas DataFrame we applied a basic trim to the data. Inspecting, from the beginning of the file, every 20 data points. If the mean acceleration over that sample was < 0.3 then the data was removed. We did this from the beginning of the file as well as the end of the file. This would help us reduce unnecessary noise at the beginning and end of the file, making segmentation easier.

Next, we began to explore the data by inspecting the raw sensor data and applying visualization techniques. This would allow us to understand the data and what features of the data would play a significant role in classification.

Figure 2: Raw gyroscopic data for a wave gesture recording.

Figure 3: Raw gyroscopic data for a circle gesture recording.

## Data Pre-Processing

To balance the dataset, ensuring that the model would not prioritise one gesture, we found which gesture had the least number of recordings and then dropped all recordings greater than that number. This step would ensure that each gesture contributed equally to the model training.

To begin our pre-processing, we used the scipy.signal butter and filtfilt functions, applying a low-pass frequency filter to the data. This filter would apply dampening to the sensor data and reduce noise. Visually, this made the waveforms smoother and less jittery. Filtering ensures that the model does not overfit the model based on all the noise in the system but uses the more generic shape of the data.

We also applied an exponentially weighted moving average (EWMA) filter at this stage. EWMA filtering was performed so that spikes in the data were dampened. This dampening was especially important when considering recordings that contained many different types of gestures as some gestures tended to have much higher acceleration than other, simply due to the nature of the gesture.

Next, we applied a scikit-learn MinMaxScaler to the data – which transforms the data to values between 0 and 1 whilst maintaining the relationships between the datapoints, this is done using the below formula:

xscaled=x-xminxmax-xmin

We performed this transformation to ensure that the machine learning models did not perceive higher values as more important for training.

Next, we applied a Fourier transformation to the data. Fourier transformation can be used to decompose a signal into its underlying frequencies, which can be useful for removing noise from the gyroscopic data as well as determining the most important parts of the signal that define the gestures.

Figure 4: Example gesture data before and after Fourier transformation and filtering.

Applying this Fourier transformation filtering in this way meant that the signals were more uniform and less prone to noise. It is hoped that a simpler wave signal will translate into a more accurate model.

Dynamic time warping (DTW) can also be used to find the similarities/distance between different wave signals, which can be used to shift rotate the waves so that slight misalignments in the time domain are not factors in the segmentation and eventual feature extraction of the gestures. DTW was considered a stretch goal for the project but was not implemented.

## Segmenting the Data

The next step of processing was to separate the individual gestures from the files that contained six to nine gestures of the same type. This would be required so that each gesture was separated in its own block of data to be associated with a label. Our first attempt to apply this separation was to visually inspect the data and its rolling average and deviation – this would help us identify an appropriate value to decide on when one gesture was starting, and another was ending.

Figure 5:Rolling standard deviation of a circle gesture recording.

This figure outlines our suggested splitting point (any points where the acceleration standard deviation is lower than the threshold, in green). Using these points, we could segment the file into its separate gestures ready for feature extraction.

An alternative approach was then adopted where we utilised scipy.signal find_peaks to automatically detect peaks in absolute acceleration, and from those peaks expand its boundaries until the acceleration was below some threshold (indicating the end of the gesture).

Figure 6: Absolute acceleration graph for a circle recording, showing the peaks in the data that represent the gestures.

We could then visually inspect the gestures to ensure that the segmentation was being correctly applied.

Figure 7: A preview of the segmented gesture data.

## Feature Extraction

For the model to accept our data we would need to transform the data once again so that each gesture’s data was the same shape. To achieve this requirement, we would segment the data, this time with each gesture being broken up into n distinct sections with some overlap. Then using these slices of data, we could extract key features such as: mean, standard deviation, median, skew and kurtosis. This would result in data that is a summary of each slice of the gesture. These features could then be visualised and taken through the feature selection process.

## Feature selection

Once all features were extracted, we visually inspected the averages of the features to find the features that had the clearest disparity between the gestures. Feature selection is critical to reduce the computational and time cost of classification, as well as increase the model’s accuracy.

Figure 8: Feature extraction/visualisation for the mean of each segment.

Figure 9: Feature extraction/visualisation for the standard deviation of each segment.

Figure 10: Feature extraction/visualisation for the kurtosis of each segment.

Originally, we had planned to use these visualisations to find features that provided clear separation between the gestures and to drop the features that did not provide strong correlation. However, this process allowed room for ambiguity and did not allow easy automation. Instead, principal component analysis was used to automate the process of feature aggregation and selection.

Figure 11: Principal component analysis visualisation for n=3.

After experimenting with various values of n, the accuracy for model training was best when the dataset was reduced to just two components.

## Train/Test/Validation Data Splitting

## Model selection and parameter hypertuning

### K-means Clustering

Write up goes here…

### Random Forest Classifier

A RandomForestClassifier was used with a predefined random state for reproducibility. Grid search was applied to fine-tune hyperparameters such as the number of estimators, maximum depth, minimum samples split, and minimum samples leaf. Cross-validation (cv=3) was used with balanced accuracy as the scoring metric.

### Long Short-Term Memory layer (LSTM)

LSTM is a recurrent neural network which is used for sequence prediction, such as involving time-series data. LSTM can capture temporal dependencies which make them effective for recognition gesture recognition tasks. In our implementation, we preprocess the data, encoded the gesture labels using LabelEncoder to convert them into numerical labels, data are then standardize  using StandardScaler. Subsequently, the number of time steps of windows representing the quantity of previous time steps of data that will be utilized to predict the current gesture is defined. The data is then prepared for model training using a custom function which generates sequences of data samples based on the time steps defined earlier.

LSTM sequential model is built using Keras which consisting of two LSTM layers with 50 neural units each followed by a Dense layer having softmax activation for multi-class classification. After that model is compiled using the Adam optimizer and sparse categorical crossentropy loss function. The model is then trained and  validated using the training and validation set.

# Results

Our model performance …

## Machine learning Models

### K-means Clustering

Despite the appearance of well-defined clusters, the model has a silhouette score of only 0.62, an accuracy of 0.31 on testing data and a validation accuracy of 0.23. Increasing the number of principle components in the data drastically reduces the accuracy of the model to almost zero. Increasing the number of clusters gave a decrease in accuracy but reducing the clusters to two gave an increased accuracy of 0.60 for testing and 0.43 for validation.

Figure 13: K-means clustering visualisation for four gestures.

### K-nearest neighbours (KNN)

Write up goes here…

### Random Forest Classifier

The best parameters obtained from the grid search were {'n_estimators': 250, 'max_depth': None, 'min_samples_split': 10, 'min_samples_leaf': 2}. The  optimal model was then used to predict gestures on the testing set. The test set achieved a balanced accuracy of 69%, thereby providing an indication of the model's overall predictive ability.

### Support Vector Machines (SVM)

The data was pre-processed using standard scaling, and a grid search with cross-validation was carried out to optimize the SVM's hyperparameters, focusing on the regularization parameter C, kernel type, and gamma value. The optimal parameters were C=0.1, using an 'rbf' kernel, and 'scale' for gamma.

After training, the SVM model achieved a balanced accuracy of approximately 54.8% on the test set, indicating its ability to generalize to unseen data. While this accuracy may be considered modest, it provides valuable insights into the classification performance of the SVM for the given gesture recognition task.

It's worth noting that the process involved substantial computational resources, and attempts were made to fine-tune the parameters by experimenting with various kernel types and C values, but these adjustments did not yield significant improvements in performance.

Overall, accuracy for SVM model has shown its potential for gesture classification, although further exploration, feature engineering, or alternative algorithms could be considered to enhance accuracy further.

### XGBoost Classifier

The XGBoost classifier was used for gesture classification on the dataset. The dataset was divided into training and testing sets, and a grid search was performed to optimize the hyperparameters of the XGBoost model. The parameters used were number of estimators (trees) in the ensemble, maximum depth of each tree, learning rate, and subsampling ratio. The best parameters identified through the grid search were an ensemble of 300 trees with a maximum depth of 20, a learning rate of 0.1, a subsampling ratio of 0.8.

After training, it achieved a balanced accuracy of approximately 68.17% on the test set. This performance indicates the model's ability to classify gestures based on the features.

In comparison to SVM, the XGBoost model demonstrated a higher balanced accuracy, suggesting its potential as a robust classifier for gesture recognition. Further exploration could involve feature engineering, or additional parameter tuning to potentially enhance classification performance even further.

## Neural Network Architectures

### Gated Recurrent Unit (GRU)

Write up goes here…

### Long Short-Term Memory layer (LSTM)

After training, model is evaluated on the test set to  which have shown promising results (and the accuracy of the model was found to be 100 percent).

| Table Head | EXAMPLE TABLE HEADER |  |  |
| --- | --- | --- | --- |
|  | Table column subhead | Subhead | Subhead |
| copy | More table copya |  |  |

# Discussions

- Limitations

- Stretch goals (DTW)

# Conclusion

Classifying gestures from mobile device sensor can be difficult due to the variation in movement that is possible from user on a mobile device.

# References

Akhavian, R. and Behzadan, A.H. (2015) ‘Construction equipment activity recognition for simulation input modeling using mobile sensors and machine learning classifiers’, Advanced engineering informatics, 29(4), pp. 867–877. Available at: https://doi.org/10.1016/j.aei.2015.03.001.

Moccia, Sara et al. “Automated Classification of Hand Gestures Using a Wristband and Machine Learning for Possible Application in Pill Intake Monitoring.” Computer methods and programs in biomedicine 219 (2022): 106753–106753. Web.

Odhiambo, Chrisogonas Odero et al. “Detecting Medication-Taking Gestures Using Machine Learning and Accelerometer Data Collected via Smartwatch Technology: Instrument Validation Study.” JMIR human factors 10 (2023): e42714–e42714. Web.

Zhao, Shenglin et al. “Hand Gesture Recognition on a Resource-Limited Interactive Wristband.” Sensors (Basel, Switzerland) 21.17 (2021): 5713. Web.

Siddiqui, Uzma Abid et al. “Wearable-Sensors-Based Platform for Gesture Recognition of Autism Spectrum Disorder Children Using Machine Learning Algorithms.” Sensors (Basel, Switzerland) 21.10 (2021): 3319. Web.

Yu, Yi et al. “Exploration of Chinese Sign Language Recognition Using Wearable Sensors Based on Deep Belief Net.” IEEE journal of biomedical and health informatics 24.5 (2020): 1310–1320. Web.

Han H, Yoon SW. Gyroscope-Based Continuous Human Hand Gesture Recognition for Multi-Modal Wearable Input Device for Human Machine Interaction. Sensors (Basel). 2019 Jun 5;19(11):2562. doi: 10.3390/s19112562. https://www.ncbi.nlm.nih.gov/pmc/articles/PMC6603535/

Stancic, I. et al. (2022) ‘Comparison and Evaluation of Machine Learning-Based Classification of Hand Gestures Captured by Inertial Sensors’, Computation, 10(9), p. 159. Available at: https://doi.org/10.3390/computation10090159.

https://phyphox.org/